Re-Upholstering 1982 Mazda RX7 Seats

IMPORTANT Note for All Steps

- : Take Lots of Pictures and Mark all the Parts with a Label (Dollar Store- Cheap Stick on)

- This was a huge help to me as I needed to reference past Pictures, as I had a Gap in Time between Dis-Assembly and Re-Assembly.

Hardware Removal

- Passenger Seat

Brackets

  - RHS- Unscrew the 4 Bolts (T50) holding the bracket containing the Seat Adjustment Handle (RHS). Remove the bracket from the Seat

  - LHS – Remove the Plastic Cap. Then remove the Cotter Pin and pull out the Plug and Plastic Washer. Remove from the Seat

Rails for bottom Seat

  - RHS-Unscrew 2 Bolts (12MM)- Rail has the Adjustment Rod for the Seat. Remove the rail from the Seat

  - LHS- Unscrew 2 Bolts (12MM) – No Rod- Remove the rail from the Seat.

- Driver’s Seat

Brackets

  - LHS- Unscrew the 4 Bolts (T50) holding the bracket containing the Seat Adjustment Handle (RHS). Remove the bracket from the Seat

  - RHS – Remove the Plastic Cap. Then remove the Cotter Pin and pull out the Plug and Plastic Washer. Remove from the Seat

Rails for bottom Seat

  - LHS-Unscrew 2 Bolts (12MM)- Rail has the Adjustment Rod for the Seat. Remove the rail from the Seat

  - RHS- Unscrew 2 Bolts (12MM) – No Rod- Remove the rail from the Seat.

Removing the Old Seat Covers

- Passenger & Driver’s Seat

  - Bottom Seat

    - Remove all Hog Rings ( 4 Rings-On 3 Sides & 3 Rings on Bracket Side) and peel off cover.

    - Inspect and replace foam as necessary.

  - Top Seat

    - Remove all Hog Rings  (5 rings on Bottom) and peel off cover .

    - Inspect and replace foam as necessary

- Note: I took my Seats to a small Upholstery Shop for Foam Repair. They filled with foam and glued the Bolsters (Drivers Seat) and one Side on the Passenger Seat (Looked like a mouse got into it—chewed up. Cheap $

  - Custom Cushion Centre- Burlington Ontario Canada (small Family Operation)

Assembling New Covers